AVL  User Primer                                 last update  3 July 1998

Mark Drela, MIT Aero & Astro

AVL (Athena Vortex Lattice) was originally written by Harold Youngren

circa 1988 for the MIT Athena TODOR aero software collection.  A number

of modifications have since been added by Mark Drela and Harold Youngren.

Input file

----------

AVL takes one input file which describes the vortex lattice geometry.

The geometry is described in the following Cartesian system:

X   downstream

Y   out the right wing

Z   up

The freestream must be at a reasonably small angle to the X axis,

since the trailing vorticity is oriented parallel to the X axis.

The input file begins with the case information in the first 5 lines:

ABCD...                    | case title, uppercase to plot properly

#                          | comment line begins with "#" or "!"

0.0                        | Mach

1     0     0.0            | iYsym  iZsym  Zsym

4.0   0.4   0.1            | Sref   Cref   Bref

0.1   0.0   0.0            | Xref   Yref   Zref

Mach  = freestream Mach number (< 1) for Prandtl-Glauert correction

iYsym =  1  case is symmetric about Y=0    , (X-Z plane is a solid wall)

= -1  case is antisymmetric about Y=0, (X-Z plane is at const. Cp)

=  0  no Y-symmetry is assumed

iZsym =  1  case is symmetric about Z=Zsym    , (X-Y plane is a solid wall)

= -1  case is antisymmetric about Z=Zsym, (X-Y plane is at const. Cp)

=  0  no Z-symmetry is assumed (Zsym ignored)

Sref  = reference area  used to define force coefficients (CL, CD, CM, etc)

Cref  = reference chord used to define pitching moment (CM)

Bref  = reference span  used to define roll,yaw moments (CR,CN)

X,Y,Zref = location about which moments and rotation rates are defined

Only the half (non-image) geometry must be input if symmetry is specified.

Ground effect is simulated with iZsym = 1, and Zsym = location of ground.

Forces are not calculated on the image/anti-image surfaces.

Sref and Bref are assumed to correspond to the total geometry.

The remainder of the file consists of a set of keywords and associated

data.  Each keyword expects a specific number of lines of data to

immediately follow it.  The keywords must also be nested properly

in the hierarchy shown below (only the first four characters of each

keyword are actually significant, the rest are just a mnemonic),

SURFACE

YDUPLICATE

HINGE

CONTROL

TRANSLATE

ANGLE

SECTION

NACA

SECTION

CAMBER

SECTION

AIRFOIL

SECTION

CFILE

SECTION

AFILE

DESIGN

SURFACE

YDUPLICATE

SECTION

SECTION

SURFACE

.

.

etc.

The YDUPLICATE, HINGE, CONTROL, TRANSLATE, and ANGLE keywords can all

be used together.  If more than one of these appears for a surface,

the last one will be used and the previous ones ignored.

At least two SECTION keywords must be used for each surface.  Also,

these must appear after any YDUPLICATE,HINGE,CONTROL,TRANSLATE,ANGLE

keywords.

The NACA, CAMBER, AIRFOIL, CFILE, AFILE keywords are alternatives.

If more than one of these appears after a SECTION keyword, the last

one will be used and the previous ones ignored.  i.e.

SECTION

NACA

CAMBER

CFILE

is equivalent to

SECTION

CFILE

*****

SURFACE            | (keyword)

Main Wing          | surface name string

12   1.0           | Nchord  Cspace

The SURFACE keyword declares that a surface is being defined until

the next SURFACE keyword, or the end of file is reached.  A surface

does not really have any significance to the underlying AVL vortex

lattice solver, which only recognizes the overall collection of all

the individual horseshoe vortices.  SURFACE is provided only as a

configuration-defining device, and also as a means of specifying

surface deflections (described later).  Also, partial forces are

calculated and displayed separately for each surface.  This is

necessary for load calculations, for example.

Nchord =  number of chordwise horseshoe vortices placed on the surface

Cspace =  chordwise vortex spacing parameter (described later)

*****

YDUPLICATE      | (keyword)

0.0             | Ydupl

The YDUPLICATE keyword is a convenient shorthand device for creating

another surface which is a geometric mirror image of the one being

defined.  The duplicated surface is _not_ assumed to be an

aerodynamic image or anti-image, but is truly independent.

A typical application would be for cases which have geometric

symmetry, but not aerodynamic symmetry, such as a wing in yaw.

Defining the right wing together with YDUPLICATE will conveniently

create the entire wing (as two separate surfaces, though).

The YDUPLICATE keyword can _only_ be used if iYsym = 0 is specified.

Otherwise, the duplicated real surface will be identical to the

implied aerodynamic image surface, and velocities will be computed

directly on the line-vortex segments of the images.  This will

almost certainly produce an arithmetic fault.

If the hinge axis for the current surface is (X Y Z), then the

the hinge axis for the duplicate surface is (X -Y Z).  The two

surfaces will then move opposite (like ailerons) when the same

hinge deflection angle is specified interactively.  Symmetric

(elevator or flap) movement is specified via hinge angles

of opposite sign.

Ydupl =  Y position of X-Z plane about which the current surface is

reflected to make the duplicate geometric-image surface.

*****

HINGE             | (keyword)

0.0  1.0  0.0    | X,Y,Z hinge-axis vector  (any scale)

The HINGE keyword defines the hinge axis about which the

surface (its normal vectors, actually) is to be deflected

interactively during program execution.  If omitted,

a default axis will be defined to extend from the first

to the last section, lying in the Y-Z plane.

*****

CONTROL           | (keyword)

2                | number of control-variable lines to follow

E  1.0           | delta_e declaration, d(hinge_deflection)/d(delta_e)

R  2.0           | delta_r declaration, d(hinge_deflection)/d(delta_r)

The CONTROL keyword declares that an additional hinge deflection for this

surface is to be governed by one or more control variables.  Up to six

different control variables can be selected, indicated by the following

key characters.

key   variable    intended use , primary control action

---   --------    -------------------------------------

A     delta_a      aileron    ,      x moment

E     delta_e      elevator   ,      y moment

R     delta_r      rudder     ,      z moment

T     delta_t      thrust     ,      x force

Y     delta_y      side force ,      y force

F     delta_f      flaps      ,      z force

The control derivatives will be generated for all variables

which are declared.

More than one variable can contribute to the motion of a surface.

In the example above, the surface total surface deflection will

be the result of two contributions on top of the basic manual

hinge deflection:

added_hinge_deflection  =  1.0 x delta_e  +  2.0 x delta_r

This can be used to simulate control mixing.  The following

is a typical definition for both halves of a V-tail, where

delta_e is the symmetric deflection the tails, and delta_r

is the antisymmetric deflection, in a 1:2 proportion.

SURFACE

Right Tail

CONTROL

2

E  1.0

R  2.0

.

.

SURFACE

Left Tail

CONTROL

2

E -1.0

R  2.0

These declarations assume that the hinge vectors point from root

to tip for each tail.  This "mirror" reflection of the hinge

vector is the definition used by YDUPLICATE, and is also the default

definition.  When the duplicate surface is created, some of the

d(hinge_deflection)/d(delta) coefficients are negated to produce

the expected action, as given by the table below.

as defined         =>        duplicated

--------------------        --------------------

d(hinge)/d(delta_a)         d(hinge)/d(delta_a)

d(hinge)/d(delta_e)        -d(hinge)/d(delta_e)

d(hinge)/d(delta_r)         d(hinge)/d(delta_r)

d(hinge)/d(delta_t)        -d(hinge)/d(delta_t)

d(hinge)/d(delta_y)         d(hinge)/d(delta_y)

d(hinge)/d(delta_f)        -d(hinge)/d(delta_f)

Hence, the definition

SURFACE

Right Tail

YDUPLICATE

0.0

CONTROL

2

E  1.0

R  2.0

is equivalent to defining both right and left sides explicitly

as in the previous example.

*****

TRANSLATE         |  (keyword)

10.0  0.0  0.5    | dX  dY  dZ

The TRANSLATE keyword allows convenient relocation of the entire

surface without the need to change the Xle,Yle,Zle locations for

all the defining sections.

dX,dY,dZ =  offset added on to the Xle,Yle,Zle values for all the

defining sections in this surface.

*****

ANGLE       |  (keyword)

2.0         | dA

The ANGLE keyword allows convenient changing of the incidence angle

of the entire surface without the need to change the Ainc values

for all the defining sections.

dA =  offset added on to the Ainc values for all the defining sections

in this surface

*****

SECTION                              |    (keyword)

0.00  0.00  0.00  0.50 1.00  5 -2.0  | Xle Yle Zle Chord Ainc Nspan Sspace

The SECTION keyword defines an airfoil-section camber line at some

spanwise location on the surface.

Xle,Yle,Zle =  airfoil's leading edge location

Chord       =  the airfoil's chord  (trailing edge is at Xle+Chord,Yle,Zle)

Ainc        =  incidence angle, taken as a rotation (+ by RH rule) about

the surface's spanwise axis projected onto the Y-Z plane.

Nspan       =  number of spanwise vortices until the next section

Sspace      =  controls the spanwise spacing of the vortices

(Nspan and Sspace for the last section in the surface are ignored)

Note that Ainc is used only to modify the flow tangency boundary

condition on the airfoil camber line, and does not rotate the geometry

of the airfoil section itself.  This approximation is consistent with

linearized airfoil theory.

The local chord and incidence angle are linearly interpolated between

defining sections.  Obviously, at least two sections (root and tip)

must be specified for each surface.

The default airfoil camber line shape is a flat plate.  Four keywords,

described below, are available to define non-planar camber lines.

If one of these is used, it must immediately follow the data line

of the SECTION keyword.

All the sections in the surface must be defined in order across the span.

*****

NACA                      |    (keyword)

4300                      | section NACA camberline

The NACA keyword sets the camber line to the NACA 4-digit shape specified

*****

CAMBER                    |    (keyword)

20                        | N

0.0   0.0                 | x/c(1)  y/c(1)

0.1   0.002               | x/c(2)  y/c(2)

.     .                  |  .       .

.     .                  |  .       .

.     .                  |  .       .

1.0  -0.01                | x/c(N)  y/c(N)

The CAMBER keyword declares that the camber line definition is input

as a set of x/c, y/c pairs.

N       =  number of points to follow

x/c,y/c =  camber line coordinates

The x/c, y/c pairs are splined, and the slope of the y(x) function

is used to modify the flow tangency condition in the same manner as

the overall incidence Ainc.  To avoid extrapolating the spline, the

x/c values should run from 0 to 1.  The y/c values do not need to

be 0 at either endpoint.

*****

AIRFOIL   X1  Y1          |    (keyword)

20                        | N

0.0   0.0                 | x/c(1)  y/c(1)

0.1   0.002               | x/c(2)  y/c(2)

.     .                  |  .       .

.     .                  |  .       .

.     .                  |  .       .

1.0  -0.01                | x/c(N)  y/c(N)

The AIRFOIL keyword declares that the airfoil definition is input

as a set of x/c, y/c pairs.

N       =  number of points to follow

x/c,y/c =  airfoil coordinates

The x/c, y/c coordinates run from TE, to LE, back to the TE again

in either direction.  These corrdinates are splined, and the slope

of the camber y(x) function is obtained from the middle y/c values

between top and bottom.  The rest of the treatment is as for CAMBER.

If present, the optional X1 X2 parameters indicate that only

the x/c range X1..X2 from CFILE is to be assigned to the surface.

If the surface is an 20%-chord flap, for example, then X1 X2

would be 0.80 1.00.  This allows the camber shape in CFILE to be

easily assigned to any number of surfaces in picewise manner.

*****

CFILE      X1  X2         | (keyword)   [ optional x/c range ]

filename                  | filename string

The CFILE keyword is essentially the same as CAMBER, except

that the x/c,y/c pairs are read from the file "filename".

The number of points N is not specified explictly anywhere.

The optional X1 X2 parameters are used as in AIRFOIL.

*****

AFILE      X1  X2         | (keyword)   [ optional x/c range ]

filename                  | filename string

The AFILE keyword is essentially the same as CAMBER, except

that the x/c,y/c pairs are generated from a standard (XFOIL-type)

set of airfoil coordinates contained in the file "filename".

The first line of this file is assumed to contain a string

with the name of the airfoil (as written out with XFOIL's SAVE

command).

The optional X1 X2 parameters are used as in AIRFOIL.

*****

DESIGN                  | (keyword)

Nline                   | number of lines to follow

Kdes  Wdes              | variable index,  variable weight

Kdes  Wdes              | variable index,  variable weight

Kdes  Wdes              | variable index,  variable weight

Kdes  Wdes              | variable index,  variable weight

.                           .

.                           .

This declares that the section angle Ainc is to be virtually

perturbed by some number of design variables, each having index

Kdes and weight Wdes.  For example, the declaration

DESIGN

3

1  3.0

4  2.0

5  1.5

specifies that the total virtual angle of the section is

Ainc_total = Ainc  +  3.0*D_1  +  2.0*D_4  +  1.5*D_5

where D_1, D_4, D_5 are design variables.

The sensitivities of the flow solution to design variable changes

can be displayed at any time during program execution.  Hence,

design variables can be used to quickly investigate the effects

of twist changes on lift, moments, induced drag, etc.

The ability to superimpose design variables for any section angle

allows the design variables to represent convenient "design modes",

such as linear washout, which influence all sections.

*****

CDCL                         |  (keyword)

CL1 CD1  CL2 CD2  CL3 CD3    |  CD(CL) function parameters

The CDCL keyword specifies a simple profile-drag CD(CL) function

for this section.  The function is parabolic between CL1..CL2 and

CL2..CL3, with rapid increases in CD below CL1 and above CL3.

See the SUBROUTINE CDCL header (in cdcl.f) for more details.

The CD(CL) function is interpolated for stations in between

defining sections.  Hence, the CDCL declaration on any surface

must be used either for all sections or for none.

****************************

Spacing Parameters -- Sspace, Cspace

These parameters must fall in the range  -3.0 ... +3.0 , and they

determine the spanwise and chordwise horseshoe vortex distributions

as follows:

parameter                              spacing

---------                              -------

3.0        equal         |   |   |   |   |   |   |   |   |

2.0        sine          || |  |   |    |    |     |     |

1.0        cosine        ||  |    |      |      |    |  ||

0.0        equal         |   |   |   |   |   |   |   |   |

-1.0        cosine        ||  |    |      |      |    |  ||

-2.0       -sine          |     |     |    |    |   |  | ||

-3.0        equal         |   |   |   |   |   |   |   |   |

spanwise:      first section        ==>       last section

chordwise:     leading edge         ==>       trailing edge

An intermediate parameter value will result in a blended distribution.

The most efficient distribution (best accuracy for a given number of

vortices) is usually the cosine (1.0) chordwise and spanwise.  If the

wing does not have a significant chord slope discontinuity at the

centerline, such as a straight, elliptical, or slightly tapered wing,

then the -sine (-2.0) distribution from root to tip will be more

efficient.  This is equivalent to a cosine distribution across the

whole span.  The basic rule is that a tight chordwise distribution

is needed at the leading and trailing edges, and a tight spanwise

distribution is needed wherever the circulation is changing rapidly,

such as taper breaks, and especially at flap breaks and wingtips.

If a surface airfoil camber line has a significant discontinuity

in the slope due to flap deflection, say, it may be appropriate

to define the surface as two separate surfaces each with its own

cosine distribution.  This will bunch points at the hinge line,

giving good accuracy.  Separate definitions are needed anyway if

the flap is to be deflected independently of the wing.

Execution

---------

AVL is executed with the input filename as an argument:

% avl filename

After the geometry input is processed and the vortex lattice is

constructed, the aero influence coefficient (AIC) matrix is factored,

and the induced velocity matrix is constructed.  The user is then

put into an operating menu which is mostly self-explanatory:

==============================================

M ach  number

R oll  rate       A lpha

P itch rate       B eta

Y aw   rate       C lear all

-------------------------------------

H inge defl.      I mage side

CO ntrol defl.

DE sign mod.      O ptions

-------------------------------------

G eometry plot    T refftz plane plot

--------------------------------------

D  stab. derivs   VB strip Vz, My

SE nsitivities    RE ference data

FT total forces   FN surface forces

FS strip forces   FE element forces

Q uit             W rite forces to file

Select action:

In general, if an operating variable (alpha, beta, roll rate, etc.)

or a surface deflection is specified, it will remain fixed at that

value until it is specified again, or cleared via the C command.

Anytime a command is given to specify an operating variable, such

as B for beta (yaw angle), the prompt will display the current value:

Enter beta (deg)  (999 to trim CN):   8.0000

Just hitting <return> will take the current 8.0 value as the default

response.

As indicated in the prompt, any operating parameter can also be

indirectly specified by requesting that the associated moment

be driven to zero.  Each moment component is associated with

a parameter as follows:

pitch moment CM  <=>  alpha, pitch rate

yaw moment CN  <=>  beta ,   yaw rate

roll moment CR  <=>          roll rate

More than one moment can be trimmed simultaneously, but a moment

cannot be trimmed "twice" --- e.g. 999 cannot be specified for

both alpha and pitch rate.

Trimming the pitching moment coefficient CM is a very easy way

to determine the trimmed CL value for each surface.  By repeatedly

deflecting the tail surface, say, a trimmed-CL versus tail setting

line can be generated very quickly.  The W command will write

all the surface deflections and surface CLs to disk for convenient

later perusal.  Each time the W command is issued, the name of the

output file will be requested, with the previous filename (if any)

shown as the default:

Enter summary output file: file.out

Just hitting <return> will result in the present conditions and

forces being appended to this file.  If another filename is entered,

the previous file will be closed and the new file will be created.

The summary output file can be read with program TRIM and plotted

(although this program is somewhat ad-hoc at present).

Another typical application for the trimmed-moment options might

be for example to determine the effect of sideslip or aileron

deflection on the steady roll rate.

Besides allowing moment trimming, the alpha command A in addition

allows the total CL to be specified:

Enter Alpha        (deg):    2.0000

CL (to specify CL):    0.8000

CM (to trim pitch)

Entering "CL" will produce the CL prompt:

Enter specified CL:   0.8000

which also offers the current specified-CL value as the default.

Output

------

Everytime an operating parameter is specified, the integrated forces

are displayed for the entire configuration and the individual surfaces.

The loads on the individual vortex elements can be displayed with the

E command, but this is rather voluminous and often not very informative.

The force and moment directions are in stability axes x,y,z, which

are tilted up by the angle alpha from the body axes X,Y,Z:

| x |     | cos(a)    sin(a)| | X |

| y |  =  |        1        | | Y |

| z |     |-sin(a)    cos(a)| | Z |

The following standard normalizations are used.

CD = F_x / (q Sref)            drag

CY = F_y / (q Sref)            side force

CL = F_z / (q Sref)            lift

CR = M_x / (q Sref Bref)       roll  moment

CM = M_y / (q Sref Cref)       pitch moment

CN = M_z / (q Sref Bref)       yaw   moment

The CD,CY,CL forces are positive in the direction of the x,y,z axes,

respectively.  Likewise, the moment components CR,CM,CN are positive

about the x,y,z axes by righthand rule.

The roll, pitch, and yaw rates (WX,WY,WZ) input from the operating

menu are defined in the body axes.  This convention was chosen for

the greatest programming simplicity rather than some aerodynamic

tradition.  The stability-axes rates Wx,Wy,Wz are easily obtained

using the  x,y,z <= X,Y,Z  transformation matrix shown above.

The reverse transformation is simply the inverse of this matrix:

| WX |     | cos(a)   -sin(a)| | Wx |

| WY |  =  |        1        | | Wy |

| WZ |     | sin(a)    cos(a)| | Wz |

It must be pointed out that if sideslip (beta) is nonzero, then

CD and CY are not the true "drag" and "side-force" aligned with

the relative wind direction.  Likewise for CR and CM.  The

wind-axes directions are given by

| x |          | cos(b) sin(b)    | | x |

| y |       =  |-sin(b) cos(b)    | | y |

| z |_wind     |                1 | | z |

| cos(b)cos(a)   sin(b)   cos(b)sin(a)| | X |

=  |-sin(b)cos(a)   cos(b)  -sin(b)sin(a)| | Y |

|      -sin(a)     0            cos(a)| | Z |

hence

CD_wind =  CD cos(b) + CY sin(b)

CY_wind =  CY cos(b) - CD sin(b)

CL_wind =  CL

CR_wind =  CR cos(b) + CM sin(b)

CM_wind =  CM cos(b) - CR sin(b)

CN_wind =  CN

AVL does not display these wind-axes forces since they are not

relevant to stability and control calculations, and differ from the

stability-axes forces only if a steady-state sideslip is present,

such as perhaps in a steady turn.  The primary quantity of interest

here is the overall L/D = CL_wind/CD_wind = CL/CD_wind, and CD_wind

is more accurately obtained from the Trefftz-Plane anyway.

The alternative Trefftz-Plane drag coefficient CDi is calculated

from the wake trace in the Y-Z plane far downstream.  This is

generally more reliable than the CD obtained from surface force

integration, and is the appropriate wind-axes induced drag for

performance prediction.

The span efficiency is defined as

2                             2

e  =  CL  / (pi A CDi)    ;   A = Bref / Sref

with Sref being replaced by  2 Sref  for Y-image cases (iYsym = 1).

Stability derivatives

---------------------

Command D generates the stability derivative matrix for the

current conditions.  The derivatives are expressed entirely

in the AVL stability axes (both the forces as well as rates).

Note that the AVL and the "traditional" stability axes differ

in having the x and z directions reversed.

Plotting

--------

The T command starts up the Treffts Plane plot menu:

======================================================

Y plot data vs Y

Z plot data vs Z

D ownwash angle toggle

L imits for plot

R eset plot limits

N umber surfaces

A nnotate plot

H ardcopy current plot

ZM zoom

U nzoom

S ize change

Select option for Trefftz plane plot (or <return>):

These plot options are self-explanatory.

The G commands starts up the geometry-plotting menu which allows

viewing of the horseshoe vortex lattice and other geometric features.

==============================================

K ey commands (viewpoint, zoom...)

H ardcopy current plot      A nnotate plot

O ptions                    S elect surfaces

CH ordline   T     CA amber        F

CN tlpoint   F     TR ailing legs  F

BO ound leg  T     NO rmal vector  F

HI nge axes  F     LO ading plot   F

Select option (or <return>):

The K command enters a sub-sub menu which allows rotation of the

aircraft to a suitable viewing angle, zooming, distortion for

perspective, etc.

------------------------------------------------

Type keys in graphics window...

L eft             R ight        (Azim. Psi  )

U p               D own         (Elev. Theta)

T ilt z axis      S et upright  (Bank  Phi  )

C lear

Z oom on curs.    N ormal size

I ngress          O utgress

H ardcopy         J ot annotations

...<space> to exit

------------------------------------------------

These commands must be typed with the cursor in the graphics window,

and their action is performed immediately.  All other menus work in

the usual text window.